Project Summary: Hybrid Monocular Distance Estimation for Autonomous Vehicle Safety

Project Context & Collaboration

- Developed a safety-critical distance estimation system for autonomous vehicles as part of a University of Chicago capstone project in collaboration with Argonne National Laboratory

- Processed and analyzed 14,800+ dashcam images (23GB+ dataset) from multiple cities (Nashville, LA, Chattanooga) stored on Google Cloud Platform

- Ground truth distances embedded in filenames (format: seq462_dist18.48_time1743693342.885320967.jpg)

- Dataset split: 12,000 training images, 1,400 validation images, 1,400 test images

Technical Architecture & Implementation

Hybrid 3-Branch Neural Network

Built a sophisticated multi-input architecture combining:

- Global Context Branch: EfficientNetB4 backbone processing full 224×224 images for scene understanding

- Local Features Branch: EfficientNetB3 backbone analyzing 64×64 cropped vehicle patches for target-specific details

- Geometric Features Branch: 10-dimensional handcrafted feature vector including:

  - Normalized bounding box dimensions (width, height, position)

  - Camera calibration features (fx=2429.865965, fy=2424.492001)

  - Physics-based distance estimates using 1.5m average vehicle height assumption

  - Angular measurements (vertical viewing angle, angular height)

Detection Pipeline

- Implemented YOLOv8-small for vehicle detection with adaptive ROI based on lane detection

- Fixed critical bug in detection logic that was selecting vehicle's own hood instead of lead vehicles

- Developed ROI filtering with aspect ratio constraints, horizontal centering, and vertical position scoring

- Achieved robust lead vehicle identification across varying road conditions

Preprocessing Pipeline

Developed comprehensive image enhancement pipeline:

- Lens distortion correction using camera calibration parameters

- Gray World white balance adjustment

- CLAHE contrast enhancement (clip_limit=2.0, tileGridSize=8×8)

- Gamma correction for shadow/highlight balance

- All preprocessing implemented in modular, reusable Python functions

Performance Results

Model Performance Metrics

- Mean Absolute Error (MAE): 1.40 meters

- R² Score: 0.897

- Within-1-meter Accuracy: 68%

- Baseline Comparison: Achieved 78% improvement over pure geometric baseline (6.50m MAE)

Distance-Stratified Performance

Implemented distance-aware evaluation recognizing that safety implications vary by range:

- Close-range (0-3m): Most challenging due to data imbalance (only 7.5% of training data)

- Mid-range (3-10m): Strong performance with distance-stratified loss weighting (5× penalty)

- Long-range (30m+): Standard loss weighting with consistent predictions

Technical Innovations & Solutions

Training Optimization

- Implemented Huber Loss (delta=1.0) for robust handling of outliers and data imbalance

- Distance-stratified loss function: 5× penalty for 0-10m, 2× for 10-30m, 1× for 30m+

- Reduced dropout (0.2-0.4) and added L2 regularization to address overfitting

- Developed checkpoint resuming and memory-efficient data loading for GCP stability

- Resolved training kernel crashes at epoch 22 through aggressive garbage collection and CUDA cache management

Infrastructure & Deployment

- Utilized Google Cloud Platform Vertex Workbench (Tesla T4 GPUs, 8 CPUs, 32GB RAM)

- Implemented memory-efficient data loaders for 23GB+ dataset handling

- Migrated seamlessly between GCP and Colab environments for training stability

- Developed modular PyTorch DataLoader architecture supporting both geometric and depth features

Real-time Inference Capability

- Designed pipeline for 15fps real-time video processing

- Supports both batch image processing and live video input

- Integrated preprocessing, detection, and prediction modules into unified Jupyter notebook structure

Key Challenges Addressed

- Data Imbalance: Only 7.5% of samples in critical 0-3m range - explored augmentation strategies (tested 6,700 additional images, though degraded performance)

- Model Stability: Resolved GCP kernel crashes through memory optimization and checkpoint management

- Camera Calibration: Handled dimension mismatches between client specifications and actual image sizes

- Validation: Conducted comprehensive ablation studies proving CNN branches provide meaningful improvements beyond geometric formulas alone

Deliverables

- Production-ready inference pipeline for batch and real-time processing

- Comprehensive evaluation framework with distance-stratified metrics

- Modular codebase with reusable preprocessing, detection, and training components

- JSON-based detection outputs with ground truth distance extraction

- Visualization tools for annotated vehicle detection and ROI analysis

Impact & Applications

- Addresses critical safety gap in autonomous vehicle perception for collision avoidance

- Provides accurate following distance estimation essential for safe autonomous operation

- Demonstrates practical fusion of deep learning with physics-based geometric priors

- Achieves publication-worthy results advancing autonomous driving safety research

Lead Vehicle Distance Estimation - Complete Pipeline Summary

PROJECT OVERVIEW

Objective: Build a production-ready inference pipeline that predicts the distance (in meters) to a lead vehicle from dashcam images or live video using a hybrid deep learning model.

Key Achievement: 1.40m Mean Absolute Error with 0.897 R² score (78% improvement over baseline geometric methods)

SYSTEM ARCHITECTURE

INPUT (Image or Video)

↓

[STAGE 1] PREPROCESSING

├─ Undistortion (alpha=0.0, crop to valid pixels)

├─ White Balance (Gray World algorithm)

├─ CLAHE (clip_limit=2.0, tile_size=8x8)

└─ Gamma Correction (gamma=1.5)

↓

[STAGE 2] LEAD VEHICLE DETECTION

├─ Lane detection (Hough transform)

├─ YOLO-small inference on preprocessed image

├─ Hood filtering (exclude bottom 9%)

└─ Lead vehicle selection via scoring (closeness, size, alignment)

↓

[STAGE 3] FEATURE EXTRACTION

├─ Full image: resize to (224, 224)

├─ Vehicle patch: crop & resize to (64, 64)

├─ Geometric features: 10 hand-crafted features

└─ Normalize all inputs (ImageNet mean/std)

↓

[STAGE 4] MODEL INFERENCE

├─ 3-branch hybrid architecture (EfficientNetB4 + MobileNetV2 + Geometric MLP)

├─ FP16 precision for speed

└─ Output: distance in meters

↓

[STAGE 5] VISUALIZATION

├─ Draw green bounding box

├─ Overlay distance label (e.g., "2.34m")

└─ NO confidence scores displayed

↓

OUTPUT (Annotated images/video + JSON predictions)

PREPROCESSING PARAMETERS (FIXED)

CONFIG = {

'preprocessing': {

'undistort_alpha': 0.0,           # Crop to valid pixels only

'clahe_clip_limit': 2.0,          # Moderate contrast boost

'clahe_tile_grid_size': (8, 8),   # 8x8 tiles

'gamma': 1.5,                     # Brighten shadows

},

'yolo': {

'model_name': 'yolov8s.pt',       # Small model (pretrained)

'confidence_threshold': 0.25,

'device': 'cuda',

'hood_exclude_ratio': 0.09,       # Exclude bottom 9%

},

'model': {

'model_path': 'best_model.pt',    # Full PyTorch model

'precision': 'float16',            # Mixed precision for speed

'device': 'cuda',

},

'inference': {

'full_img_size': (224, 224),

'patch_size': (64, 64),

'normalize_mean': [0.485, 0.456, 0.406],

'normalize_std': [0.229, 0.224, 0.225],

},

'video': {

'target_fps': 15,

'codec': 'mp4v',

},

'camera': {

'avg_car_height': 1.5,            # Meters (for geometric features)

}

}

CAMERA CALIBRATION INPUT

User provides:

- fx: Focal length in x direction (e.g., 2429.865965)

- fy: Focal length in y direction (e.g., 2424.492001)

- dist_coeffs: Array of 5 distortion coefficients [k1, k2, p1, p2, k3]

Pipeline calculates:

- cx, cy: Principal point (image center after preprocessing)

- New camera matrix after undistortion

MODEL ARCHITECTURE

3-Branch Hybrid Model

Branch 1: Full Image (EfficientNetB4)

- Input: (224, 224, 3)

- Backbone: EfficientNetB4 (pretrained, frozen)

- Output features: 128D

Branch 2: Vehicle Patch (MobileNetV2)

- Input: (64, 64, 3)

- Backbone: MobileNetV2 (pretrained, frozen)

- Output features: 128D

Branch 3: Geometric Features (Custom MLP)

- Input: 10 hand-crafted features

  - bbox_width / img_width

  - bbox_height / img_height

  - bbox_y1 / img_height

  - bbox_y2 / img_height

  - bbox_center_x / img_width

  - bbox_center_y / img_height

  - bbox_aspect_ratio

  - distance_estimate_norm (from camera intrinsics)

  - vertical_angle_norm (from camera intrinsics)

  - angular_height_norm (from camera intrinsics)

- Output features: 16D

Fusion Head

- Concatenate: [128, 128, 16] → 272D

- Dense layers with BatchNorm, Dropout

- Final output: 1 (distance in meters)

KEY PROCESSING STEPS

1. Preprocessing Function

Input: Image + camera intrinsics

├─ Undistort using cv2.undistort (alpha=0.0)

├─ White balance (Gray World algorithm)

├─ CLAHE enhancement (clip_limit=2.0, tile=8x8)

├─ Gamma correction (gamma=1.5)

└─ Output: Preprocessed image + new camera matrix

2. Lead Vehicle Detection

Input: Preprocessed image

├─ YOLO-small inference (conf_threshold=0.25)

├─ Filter vehicle classes [2, 5, 7, 3]

├─ Remove hood detections (bottom 9%)

├─ Score detections: closeness(0.7) + size(0.2) + alignment(0.1)

└─ Output: Bounding box [x1, y1, x2, y2] of lead vehicle

3. Feature Extraction

Input: Preprocessed image + bbox + camera matrix

├─ Full image: resize → normalize (ImageNet)

├─ Vehicle patch: crop from bbox → resize → normalize

├─ Geometric features: compute 10 features from bbox + camera matrix

└─ Output: 3 tensors for model input

4. Model Inference

Input: 3-branch tensors (in FP16)

├─ Forward pass through hybrid architecture

└─ Output: Distance prediction (meters)

5. Visualization

Input: Original image + bbox + distance

├─ Draw green rectangle (4px thickness)

├─ Overlay distance label (e.g., "2.34m") at top-left

└─ NO confidence scores displayed

INPUT/OUTPUT MODES

Mode 1: Batch Image Processing

Input: List of image files

Process: Sequential (one image at a time)

Output per image:

├─ 01_preprocessing.jpg (after all preprocessing stages)

├─ 02_detection_bbox.jpg (with bounding box)

└─ 03_final_output.jpg (with distance label)

Batch output:

└─ batch_predictions.json

{

"image_001.jpg": {"distance": 2.34, "confidence": 0.92},

"image_002.jpg": {"distance": 15.67, "confidence": 0.88}

}

Mode 2: Live Video Processing

Input: Video file (MP4, AVI, MOV)

Process: Frame by frame (decode all frames at target fps)

Real-time: Display cv2.imshow for each processed frame

Output:

├─ annotated_video.mp4 (all frames with bbox + distance)

└─ video_predictions.json

{

"frame_000": {"distance": 2.34, "timestamp": 0.0, "confidence": 0.92},

"frame_001": {"distance": 2.38, "timestamp": 0.067, "confidence": 0.89}

}

CRITICAL DESIGN DECISIONS

Detection on Preprocessed Image ✓

- Why: Undistortion, white balance, CLAHE, and gamma correction enhance image quality

- Benefit: YOLO detection is more accurate on enhanced images

- Implementation: Run YOLO on preprocessed frame, extract bbox, use for feature extraction

No Confidence Score Display ✓

- Why: Focus on distance prediction only

- Implementation: Calculate but don't render on output images

Sequential Image Processing ✓

- Why: Memory efficiency for large batches

- Implementation: Process one image at a time, save outputs immediately

FP16 Precision ✓

- Why: 2x faster inference with minimal accuracy loss

- Implementation: model.half() after loading

Skip Frames with No Vehicle Detection ✓

- Why: Prevent false predictions

- Implementation: Don't include in JSON if detection fails

DEPENDENCIES & IMPORTS

# Core

import torch

import torch.nn as nn

from torchvision import models

import cv2

import numpy as np

import json

import os

# Detection

from ultralytics import YOLO

# Local modules

from preprocessing import preprocess_image, undistort_image

from lead_vehicle_detector import LeadVehicleDetector

# Optional (for UI)

import streamlit as st

import matplotlib.pyplot as plt

FILES REQUIRED

- preprocessing.py (MODIFIED)

  - Functions: preprocess_image(), undistort_image(), apply_white_balance(), apply_clahe(), apply_gamma_correction()

  - Accepts parameters instead of reading from config.py

- lead_vehicle_detector.py (ORIGINAL)

  - Class: LeadVehicleDetector

  - Methods: find_lead_vehicle(), lane detection, scoring

- best_model.pt (USER PROVIDED)

  - Trained model checkpoint

  - Format: dict with 'model_state_dict' key or direct state dict

- yolov8s.pt (DOWNLOADED)

  - YOLOv8-small pretrained weights

  - Auto-downloaded on first use

INFERENCE NOTEBOOK CELLS (16 TOTAL)

| Cell | Purpose |
| --- | --- |
| 1 | Imports & setup |
| 2 | Configuration |
| 3 | Model architecture definition |
| 4 | Load YOLO & distance models |
| 5 | Camera matrix initialization |
| 6 | Geometric features extraction |
| 7 | Preprocessing wrapper |
| 8 | Feature extraction pipeline |
| 9 | Model inference (FP16) |
| 10 | Visualization (draw bbox + distance) |
| 11 | Image batch processing |
| 12 | Video processing |
| 13 | User input for camera parameters |
| 14 | Choose input mode & process (Images) |
| 15 | OR process video |
| 16 | View results |

STREAMLIT UI (app.py)

Features:

- ✓ Sidebar for camera calibration input (fx, fy, dist_coeffs)

- ✓ Input mode selector (Batch Images or Video)

- ✓ Upload images or video

- ✓ Process with progress tracking

- ✓ Real-time display of results

- ✓ Tabbed interface (Final Output, All Stages, Predictions JSON)

- ✓ Download predictions as JSON

- ✓ Display video output

- ✓ Model caching for fast performance

Run:

streamlit run app.py

PERFORMANCE METRICS

- Speed: ~50-100ms per frame (on CUDA GPU)

- Memory: ~4-6GB GPU memory for batch processing

- Accuracy: 1.40m MAE, 0.897 R² score

- Detection Rate: 95%+ lead vehicle detection (from ablation studies)

- Precision: FP16 mixed precision (no significant accuracy loss)

ERROR HANDLING

| Scenario | Action |
| --- | --- |
| No vehicle detected | Skip frame, mark as not detected in JSON |
| Invalid camera parameters | Raise error, ask user to verify |
| CUDA unavailable | Fall back to CPU (with warning) |
| Corrupted image/frame | Skip with warning, continue processing |
| Model loading failure | Raise error with helpful message |

USAGE EXAMPLES

Jupyter Notebook (Batch Images)

image_paths = ['/path/to/image1.jpg', '/path/to/image2.jpg']

predictions = process_image_batch(

image_paths, fx, fy, dist_coeffs,

detector, yolo_model, distance_model,

output_dir='./outputs'

)

Jupyter Notebook (Video)

predictions = process_video(

'/path/to/video.mp4', fx, fy, dist_coeffs,

detector, yolo_model, distance_model,

output_dir='./outputs'

)

Streamlit UI

streamlit run app.py

# Then:

# 1. Enter camera intrinsics in sidebar

# 2. Choose input mode

# 3. Upload images/video

# 4. Click "Process"

# 5. Download results as JSON

FUTURE IMPROVEMENTS

- [ ] Temporal smoothing (exponential moving average for video)

- [ ] Batch inference for multiple images simultaneously

- [ ] Model quantization (INT8) for edge deployment

- [ ] WebRTC for real-time streaming

- [ ] REST API endpoint for integration

- [ ] Multi-camera support

- [ ] Depth estimation fallback

- [ ] Confidence calibration

KEY LEARNINGS

- Distance-Weighted Loss: Absolute MAE doesn't tell the complete story. 1.9m error at 3m (63% relative error) ≠ 1.9m at 30m (3.3% relative error). Need safety-critical close-range focus.

- Hybrid Architecture Superiority: CNN features + geometric priors > pure vision or pure geometric methods. 3-branch fusion captures complementary information.

- Preprocessing Matters: Correct camera calibration, distortion correction, and enhancement significantly improve detection and inference accuracy.

- Detection on Enhanced Images: Running YOLO on preprocessed images (undistorted, white-balanced, CLAHE enhanced) improves detection robustness.

- Data Imbalance Impact: Only 7.5% of training data in critical 0-3m range; 51.8% in 10-20m range. Requires strategic augmentation and loss weighting.

CONTACT & SUPPORT

Project: UChicago Capstone + Argonne National Laboratory Model Version: HybridDistanceEstimator (EfficientNetB4 + MobileNetV2)

NEW  ARCHITECTURE
EfficientNet Hybrid Architecture Summary

Your model uses a 3-branch hybrid approach combining EfficientNetB4 (full scene), EfficientNetB3 (vehicle patch), and geometric features.

Why EfficientNet:

- Superior accuracy-efficiency tradeoff vs MobileNet (~15-20% better accuracy on same parameters)

- Scales depth/width/resolution optimally for different input sizes

- B4 handles 224×224 full images, B3 handles small 64×64 patches efficiently

Branch 1 - Full Image (EfficientNetB4): Frozen pretrained backbone (1792 features) → Dense layers (512→256→128) with BatchNorm + Dropout at each step. Captures global scene context.

Branch 2 - Vehicle Patch (EfficientNetB3): Frozen pretrained backbone (1536 features) → Dense layers (256→128→64) with BatchNorm + Dropout. Extracts local vehicle details from cropped patch.

Branch 3 - Geometric Features: Raw hand-crafted features (7 dims: bbox dimensions, positions, camera calibration) → Dense layers (64→32→32). Provides physics-based distance hints.

Fusion Head: Concatenates all 3 branches (128+64+32=224 dims) → 4 Dense layers (256→128→64→32) with L2 regularization, BatchNorm, and adaptive dropout. Learns complex distance relationships.

Key Improvements:

- L2 regularization (1e-4) on all dense layers prevents overfitting on imbalanced data

- BatchNorm after every layer stabilizes training and allows higher learning rates

- HuberLoss (delta=1.0) instead of MSE makes model robust to outliers and emphasizes close-range accuracy (safety-critical <3m)

- Selective unfreezing: frozen backbones first (fast convergence), then unfreeze with 10x lower LR for fine-tuning

- Mixed precision training (FP16) reduces memory usage and speeds up computation

Performance: Achieves ~1.58m MAE with better generalization on underrepresented distance ranges, especially critical close-range detection.